ATARI HARD DRIVE - how to use your PC as a XL/XE hard disk.

version 1.0

Project:

Tomasz S.Piotrowski & Konrad M.Kokoszkiewicz

Hardware & PC software:

Tomasz S.Piotrowski

XL/XE system software development:

Konrad M.Kokoszkiewicz

Documentation:

Tomasz S.Piotrowski & Konrad M.Kokoszkiewicz

English translation:

Konrad M.Kokoszkiewicz

First release:

24.III.1994.

***

IMPORTANT NOTICE:

The package called "PIO2PC.*" may be freely distributed and  used

as long as any file included by the Authors won't be changed   or

removed.    We   take   no   responsibility   for    unauthorized

modifications.  Any questions,  suggestions or bug reports please

send by E-mail to:

Konrad M.Kokoszkiewicz

conradus@plearn.edu.pl

or:

conradus@plearn.bitnet

or to:

Tomasz S.Piotrowski

tomaszek@plearn.edu.pl

or:

tomaszek@plearn.bitnet

***

The archive should contain:

ATARI.PAS     -  PC handler (source code)

MAKEPART.PAS  -  The Partition Creator (PC source code)

ATARIROM.BIN  -  upgraded OS for the XL/XE computers

HD_CARD.TIF   -  PC card scheme

H_SW.TIF      -  XL/XE interface scheme

HD_PL.DOC     -  doc file in Polish (Klub Atari)

HD_ENG.DOC    -  doc file in English (this file)

***

Tomasz S.Piotrowski

Hardware documentation

The  HD_CARD.TIF file includes a scheme of the interface to  make

fast,  parallel  data transfers with automatic  acknowledgements.

On the second scheme (H_SW.TIF) you can see a simple device, that

should be mounted into your Atari. This device is responsible for

switching on/off the EPROM, that includes the device handler.

How does it work?

As the "old" Atarians certainly know,  the XL/XE operating system

handles user-own devices,  that may be connected directly to  the

bus.  The handler must be present in the FP-package area  ($D800-

$DFFF).  The  PDVREG register,  that manages both ROM  banks,  is

fixed  at $D1FF.  By setting its most significant bit the OS  can

replace the FP-package with our handler.

I  used  a trick to simplify the device.  Instead of  an  address

decoder  for the FP area,  there is the 32K EPROM  (27256),  that

replaces  the  original  Atari OS ROM.  Lower  16K  contains  the

original OS;  upper 16K contains the same, but the FP-package has

been  replaced  with  the 'new-device'  handler.  Both  ROMs  are

managed by a signal connected to the A14 pin of the EPROM  (small

scheme).  '-CE1' is a signal from I/O area decoder mounted on the

main board (U2 - 74LS138 - pin 14).  When the CPU accesses to I/O

page $D1,  there is the low level signal on this pin.  Except the

register  mentioned  above,  there are  all  (i.e.  both)  device

registers on this page.  There is the data register at $D100  and

the control register at $D140.

The interface is provided to join the XL/XE and a PC using  their

buses.  Its  base elements are 8-bit  latches  (74HCT574),  which

create  the data register,  and the 74HCT74 and  74HCT125  chips,

which create the control register. The rest are address decoders.

The data register operates bidirectionally. Data written by Atari

may be read by PC only,  and data written by PC may be read  only

by  Atari.  On  the scheme the PC decoder has  been  marked  with

fragmented  line.  If you want to build an interface for  another

computer,  only this decoder will be different; the rest won't be

changed.  The  PC data register is fixed at $300 and the  control

register at $301.  Both addresses are fixed at the prototype card

area,  so  don't  care with any  hardware  collision  (e.g.  with

another card).

When  a byte is being sent by Atari,  it writes this byte to  the

data register ($D100 - Atari).  This action sets the 6 bit in the

PC control register ($301 - PC),  so PC can read the byte sent by

Atari.  If PC do it, the 6 bit in Atari control register ($D140 -

Atari) is set automatically,  so Atari can send a next byte  etc.

If the data are sent from PC to Atari,  the transfer protocol  is

very similar.  Only difference is,  that bit 7, not 6, is used in

both control registers. Bit 6 manages transfers from Atari to PC,

bit  7 - from PC to Atari.  If the control bit  is  cleared,  the

computers should wait, if is set - they may send a byte.

The PC software emulates an Atari hard disk.  All the  partitions

are  in fact the MS-DOS files (up to 16MB each),  which  you  can

create  using the special program included in  the  archive.  You

also can create partitions on floppies,  use disk compressor like

DoubleSpace, move partitions to an archive, make backups etc. All

you need is good DOS system for Atari, like MY-DOS or SpartaDOS.

To physically connect the computers I used a printer-style  cable

about  100 cm length.  You should beware and don't use  too  long

cable,  because  it  may cause  troubles.  To  minimize  possible

transfer  disturbances,  all the lines have been connected to  PC

through  4.7k resistors.  On the another side the  interface  has

been connected directly to buffered (!) XL/XE bus.

***

Konrad M.Kokoszkiewicz

Software documentation

The XL/XE handler,  called ATARIHAN.M65,  has been written  using

The  Our5oft Macro Assembler and compiled on the  Falcon030.  The

handler uses the SIO-style transfer protocol:

command-acknowledge-data-acknowledge

For example:

DOS requests a sector to be read from a disk:

Atari          direction    PC

------------   ----------   ------------

1. CFB command   ---->      4 bytes

2. 1 byte        <----      acknowledgement

3. 256 bytes     <----      data block

4. 1 byte        <----      acknowledgement

DOS requests a sector to be written to a disk:

Atari          direction    PC

------------   ----------   ------------

1. CFB command   ---->      4 bytes

2. 1 byte        <----      acknowledgement

3. data block    ---->      256 bytes

4. 1 byte        <----      acknowledgement

DOS requests the disk status:

Atari          direction    PC

------------   ----------   ------------

1. CFB command   ---->      4 bytes

2. 1 byte        <----      acknowledgement

3. data block    <----      4 bytes

4. 1 byte        <----      acknowledgement

To  accelerate the data transfers,  the sector length  have  been

fixed at 256 bytes.  The handler don't provides any checksums for

the same reason.

The command is standard CFB block calculated from DCB  variables.

It contains the bytes, as follows:

1.  CDEVIC  -  DDEVIC ($0300) ORed with DUNIT ($0301).  It is the

physical device ID.

2. CCMND  -  is the same as DDEVIC ($0302). It is proper command.

3. CAUX1  -  the same as DAUX1 ($0309).

4.  CAUX2  -  the same as DAUX2 ($030A). Both bytes are usually a

sector number.

The acknowledgement is one byte, as follows:

1. "A"  -  Command is valid and will be executed.

2. "N"  -  Invalid command. It terminates the process immediately

and returns status 139 (Negative acknowledge) to the DOS.

3.  "C"   -  Transfer complete,  no errors.  It finishes the  I/O

session and returns status 1 (Success) to the DOS.

4. "E"  -  Invalid data block.  It finishes the I/O session  and

returns status 144 (Device done error) to the DOS. It may happen,

for example,  if the command is valid, but the PC cannot read any

sector due to damaged disk.

5.  "B"   -  Invalid device ID.  It happens,  if the  PC  doesn't

handle the device you called at all (e.g.  a printer or  standard

floppy drive).  It doesn't cause any error, the procedure returns

to the SIO immediately to allow it to call another device.

The handler is initialized by a special procedure executed by the

system  during booting.  It happens only if the PC is  physically

connected,  so you can still use the Atari even if you cannot use

your  PC at this time.  If the PC is connected,  switched on  and

ready,  and  you  want  not to use its  hard  disk,  you  needn't

disconnect.  The  initializing routine may be passed  by  holding

down  the  SELECT  key (on Atari keyboard) during  warm  or  cold

start,  so you can easy switch off the hard disk for a while  and

boot  up  from a floppy.  You can always  reinitialize  the  hard

drive, just press RESET key.

Additional feature:

Even if a PC isn't connected,  you can perform the COLD START  at

any time. Just hold down the SHIFT key, then press RESET.

*** WARNING! ***

The PC software must run before the Atari will boot up from  hard

drive.  Don't break the PC handler before you'll finish your work

on the Atari, otherwise it may hang up!