Object Time Base recovery

Contribution to the MPEG/IETF meeting

Meeting hosted by Columbia University

New-York, 25th of April 1999

D.Curet, M.Veillard, (FT)

# 1 introduction

The Object Time Base recovery presented here comes from an MPEG experience, and needs revision to take into account IETF requirements.

# 2 Abstract

After presenting some key points of the MPEG timing model, we are going to highlight the approach which was taken by MPEG2, when MPEG2 cared being carried over ATM. The result of that approach was the definition of a “ Real Time Interface ”.

If applicable such an approach could be common to MPEG4 and IETF.

# 3 timing model:

The MPEG timing model is based on a constant end to end delay which embed a variable delay (the delay experienced through each of the encoder or decoder buffers) and a constant transmission delay.

Since the end to end delay is constant, Audio and Visual compositions are precisely synchronised.

MPEG defines a System Decoder Model (SDM) which is a virtual decoder. The construction of System bitstreams is constrained such that when they are decoded by a decoder following the SDM, with correctly sized decoder buffers, those buffers will never overflow nor underflow.

According to the bitrates that can be expected with some video profiles, some applications will require ‘lip synchronisation’ for audio/video presentation.

Decoders, which are implemented in accordance with this timing model, will produce precisely timed output. Decoder implementations are not required by MPEG to decode and compose Access Units in accordance with this model. In such implementations, synchronisation may not be very precise, and decoder buffers may overflow or underflow.

In order for the decoder to deal with the precise amount of data, that causes the global end to end delay to be constant, it is necessary for the decoder to have a system time base whose frequency and absolute instantaneous value match those of the encoder.

The Object Time Base clock used at the encoder side can be either an internal clock or an external clock:

- Using an external clock can be quite straightforward when this external clock is available on both sides. The correct designation of the common external clock, and the way to come to an agreement on the use of that common clock has to be specified. The external common clock definition could be part of the SL_configuration, and could be done on an “external common clock” profile basis. In this case there is no real OTB recovery.

- Using an internal clock is more difficult, as an accurate replica of the local OTB clock has to be constantly reconstructed. In this case there is a need for an OTB recovery.

# 4 Object Time Base recovery

Object Time base recovery can be done with well known Phase Lock Loop (PLL) mechanisms.

These PLL allow to build a replica of the original Object Time Base Clock, with its characteristics:

- accuracy [worst case frequency deviation from the nominal , in parts per million],

- long term drift[worst case frequency slew rate, in ppm/s],

- short term jitter suitable for the application [peak to peak uncorrected network timing jitter, in ms or ms or s].

As soon as a decoder begins to receive and decode data, until the OTB replica is locked to the time and clock frequency that is represented by the received OCRs, data is arriving at a different rate than expected, and decoder buffer fullness will either decrease or increase. Decoder needs additional buffering.

We can have a look at the solutions and figures that MPEG took for the MPEG2 transport application:

- program clock resolution of 27 MHz clock

- a program clock accuracy of 30ppm

- an associated rate of change in time of 75*10-3 Hz/s.

- Program clock samples sent at least every 100ms.

- Program clock samples accuracy within +/- 500ns bounds.

Such figures were agreed, as:

- From the encoding and multiplexing point of view they are quite easy to satisfy.

- From a remultiplexing point of view, as a remultiplexing equipment can update the PCR samples, without locking its internal clock to each Program clock (as the PCR samples will remain within the acceptable jitter). This clock of the remultiplexing equipment being, by itself, sufficiently accurate.

- From the STB designer point of view, a simplification constraint was added on the necessary locking of all clocks (program clock, video sampling clock, audio sampling clock and multiplex clock), thus avoiding to have several PLL at the receiver side.

- Applications which need to generate a chroma sub-carrier that is compliant with NTSC specification (accuracy of 3ppm, short term jitter of 1ns, and long term drift of 0.1 Hz per second) could use such a system clock, and such jittered samples for that purpose.

Physical links have to pay much attention on the program clock sample accuracy they introduce (as with DVB ASY interfaces)

# MPEG interfaced to real transport networks

The drawing in the following page is an attempt to clarify some possible configurations where MPEG2, MPEG4, RTP and RTCP appear, and where “Real Time Interfaces” could take place.

## 5.1 The MPEG2 “Real Time Interface”:

For the MPEG virtual System Decoder Models (SDM), a non-jittered transport network is assumed. As the transport over ATM became a key issue for MPEG2 transport signals (188= 4 *47), a discussion took place on the definition of such an interface that could reflect the real world. Figures were proposed for the transport network jitter that could be expected from an AAL. The consensus could only be reached on a 50ms jitter. That interface is called “Real Time Interface”

MPEG4 could follow the same process, if applicable:

Normalised virtual OTB clocks can be stated (on an internal clock profile basis), with:

Their resolution,

Their accuracy,

Their slew rate,

A maximum interval between OCR samples.

The System Decoder Model OCR samples tolerance has also to be stated: the OCRsdm tolerance. From the MPEG2/MPEG4 adaptator, such a tolerance could be a super set of the already normalised MPEG2 PCR tolerance of +/- 500ns.

## 5.2 An “RTP/RTCP Real Time Interface”:

MPEG4 and IETF could define an interface called “RTP/RTCP Real Time Interface” which could be part of some interface profiles. Such an interface should be built on an enhanced System Decoder Model: the RTP/RTCP System Decoder Model. Such a model would accept a wider tolerance on the OCR samples: the OCRrtp tolerance.

# 6 Conclusion:

We have seen that the OTB reconstruction is part of the MPEG4 domain, but is also part of a common IETF/MPEG4 workplan. The MPEG4/RTP-RTCP interfacing, by means of a new ‘Real’ System Decoder Model definition is one of the issues.

The way an external common clock can be attached has also to be specified.

MPEG could take advantage to formalise clock and interface definitions using “external common clock”, “internal clock” profiles and “Real Time Interface” profiles.

Where to insert Real Time Interfaces?