�������  ���

�������  ���

��   �   ��    ����  �� ���  �� ���  ��  ��

����     ��   ܰ���� ��ܰ��� ��ܰ��� ��  ��

����     ��   ��  ��  ��  ��  ��  �� ��  ��

��       ��   ��  ��  ���ܰ�  ���ܰ� ���ܰ�

ܰ��      ���  ���ܰ�  �����   �����   �����

����      ���   ����  ܰ�     ܰ�      ��ܰ�

���     ���      �����

������                  ���

������                  ���

� �� �   ����    ����    ��

��    ܰ����  ܰ����   ��

��    ��  ��  ��  ��   ��

��    ��  ��  ��  ��   ��

ܰ��   ���ܰ�  ���ܰ�   ���

����    ����    ����    ���

���  ��   ��     �

���  ��   ��    ܰ

�� ܰ�  ���   ܰ��

��ܰ�   ���   ����

����     ��    ��

�����    ��    �� �

ܰ� ���   ���   ��ܰ

���  ��   ���    ��

��          ��

ܰ�         ܰ�

���         ���

��          ��

��          ��

��    ��    ��

�ܰ���  ��  �ܰ���

������  ��  ������

William Luitje

13 March 1995

This is a collection of 8 small utilities which will help you use

your  floppy  disks more effectively.   Since you probably  worry

more  about  how to get better speed and storage out of your hard

disk,   this  may  not  be a matter of pressing concern  to  you.

However,  floppies are going to be used for quite a while and  by

spending  a few minutes with these utilities you can make  things

go a  little bit faster each time you use a floppy.  Programs are

provided to

� set the mechanical parameters of operation for your floppies

� test for the maximum reliable step rate of a floppy drive

� test a drive or disk for read errors

� measure the seek performance of a floppy system

� determine the optimal skewing parameters for formatting disks

� test the rotational speed of a drive for accuracy

� show the formatting parameters for a floppy disk

� aid in the use of a head cleaning disk

�����������������������������������������������������������������Ŀ

���� FLOPPARM �����������������������������������������������������

�������������������������������������������������������������������

Floppies  have  been  around a long time and have  been  improved

since they first came out, not only in capacity but also in their

mechanical  speed and  accuracy.  Unfortunately,  system software

can't tell what the  best speed to drive a floppy is and must  be

compatible  with  old  drives  and  so uses  conservatively  slow

parameters.    When  accessing  a  floppy,  the BIOS uses a table

called  the  Diskette  Parameter  Table  for  these   parameters.

Interrupt vector 1E contains the address of this table.  Since it

is  usually  set up in RAM by DOS,  it is easy to modify  it  for

modern, fast drives.  FLOPPARM will do this for you conveniently;

simply run it  with one of the three arguments SLOW,  MEDIUM,  or

FAST.  The parameters which are affected are:

� step  rate and head unload time, which  are parameters of the

disk controller chip.

� head settle time,  which is the amount of time the BIOS takes

after stepping from one track to another to wait for the head

mechanism to stop vibrating.

� motor startup time,  which is the amount of time taken by the

BIOS  to allow the spindle motor speed to stabilize after  it

first starts up.

SLOW is provided for compatibility with really old drives; MEDIUM

is somewhat faster than DOS 5;  and FAST is pedal to metal!  When

run, FLOPPARM displays the parameters used.  If run with no or an

illegal  argument,   FLOPPARM will display a usage hint  and  the

current values of the parameters.

FLOPPARM also allows you to specify your own parameters by  using

the  CUSTOM  argument  followed by an equals sign  and  the  four

parameters  (step rate,  head unload time,  head settle time  and

motor startup time) separated by commas (no spaces allowed).  For

example a command equivalent to FLOPPARM MEDIUM would be

flopparm custom=13,15,1,2

Legal values for step rate and head unload time are from 1 to 15.

Legal values for head  settle  time  (in milliseconds)  and motor

startup time (in 1/8s of a second) are from 1 to 255.

How do you decide what custom values to use?  Obviously, to speed

up  operation,  increase step rate and decrease head unload time,

head  settle  time and motor startup time.   Since motor  startup

time is given in eighths of a second and the other parameters are

given  in milliseconds,  it seems like a good place to look for a

speed increase.   I've had no problems setting it to one and that

produces  a noticeable reduction in the time you have to wait for

the floppy when the motor is off.

I've  provided two utilities to help in testing your settings for

the rest of the parameters, TESTSTEP and FLOPTEST.  These will be

described in subsequent sections.   In any case,  once you decide

which  parameters to use,  just put a line invoking  FLOPPARM  in

your  AUTOEXEC.BAT  file  and enjoy a modest increase  in  floppy

speed.

NOTES: FLOPPARM is not a TSR and does not take up any DOS memory;

it  simply  modifies  a table in RAM which is already  created by

DOS.   Also,  there is only one table to control the operation of

all  of  the  floppy drives in your system;  make  sure that your

slowest drive can handle the parameters you set.

�����������������������������������������������������������������Ŀ

���� TESTSTEP �����������������������������������������������������

�������������������������������������������������������������������

Use TESTSTEP to find the fastest step rate which can be used with

a  particular floppy drive.  It starts with the highest  possible

step  rate,   15,  and then does a large number of seeks  on  the

selected drive to see if that rate is too fast.   If so, then the

step  rate is lowered and the test retried until all of the seeks

can be done without error.

To run this test you must use  a floppy  which has been formatted

with no errors at the standard size for that drive.  For example,

if  you are testing a 5�"  High Density drive you must use a  1.2

Meg disk,  not a 360K disk.  Similarly,  you shouldn't use a 720K

disk in a 3�" High Density drive or vice versa.

This  test appears to be very stringent;  no drive which  I  have

tested  has  passed it with a step rate of 15.   However,  during

normal  use  most floppies seem to work at that  rate,   probably

because the seeks are not as severe.   You can be sure that if it

passes this test it will work reliably during normal use.

Note that this program by doesn't test the head unload  and  head

settle  times  at  all.   You can test these indirectly with  the

FLOPTEST utility described in the next section.

TESTSTEP takes one argument,  the drive letter.  Only A and B are

supported at this time.  Here is an example command and result.

teststep b

Test Maximum Floppy Step Rate, Ver 1.0 by luitje@m-net.arbornet.org

Drive B: has 80 tracks; current Step Rate is 13

Testing Step Rate = 15:  failed

Testing Step Rate = 14:  failed

Testing Step Rate = 13:  passed

�����������������������������������������������������������������Ŀ

���� FLOPTEST �����������������������������������������������������

�������������������������������������������������������������������

FLOPTEST  provides a practical test of whether you have set  your

floppy  drive parameters too fast.   It will simply seek to every

track of the disk,  read a sector and report any errors it finds.

The seek pattern is called a "butterfly test"  since it reads the

first track, then the last track, then the second track, then the

next  to  last track,  etc.   This will really give the  drive  a

workout!

This test requires a formatted floppy which has no  bad  sectors.

(FLOPTEST  can't tell if an error is due to the drive not working

properly  or  due to a bad disk).   It is a read only test so  it

shouldn't change any data on the disk.  There are two parameters:

one is the drive letter and the second is the number of passes to

run.  If you get any errors and your floppy is ok then you should

back  off  one or more of the floppy drive  parameters  and  test

again. Here is an example

floptest a 20

If you are fortunate enough to have more than 2 disk drives (this

applies  both  to  FLOPTEST and to the other  utilities  in  this

package, except where noted), you can refer to the third drive as

c and the  fourth drive as d,  even if you have a hard drive that

goes by either of those names.  I know this is confusing but this

test operates at the  BIOS  level and has no idea what letter DOS

has assigned to your third and fourth floppies.   I  had a choice

of  confusing  people  who  had  more than  2  floppy  drives  or

confusing everybody by making you specify the drive with a number

rather than a letter.  That's life.

You  can  also use FLOPTEST to see if a floppy disk has  any  bad

sectors.    If  you run it with only the drive letter  specified,

then  it will attempt to read every sector  (linearly,   starting

with track 0) and report any failures.  Since FLOPTEST makes only

one attempt to read each sector,  it will warn you if a sector is

getting  weak.   I  use this feature a lot before putting any new

information  on an old disk.   If the disk has errors,  I  either

reformat it or throw it away.

�����������������������������������������������������������������Ŀ

���� SEEKBNCH �����������������������������������������������������

�������������������������������������������������������������������

Once  you have your parameters set as you like them you  can  use

SEEKBNCH  to  measure  how fast your drive  will  seek  with  the

controller  set  to  the  current  step  rate parameter. SEEKBNCH

performs three benchmarks:

� track to track seek time

� average seek time

� full stroke seek time

The  step  rate should be set either by default or  via  FLOPPARM

before this test is run (head settle time is ignored).   However,

do  not use a step rate faster than that passed by TESTSTEP.   At

best  the  results  will be meaningless and at  worst  you  might

damage your drive.

To  run  the  benchmark  you will need  an  error-free  formatted

floppy.  SEEKBNCH takes one parameter,  the drive letter (only  A

and B are supported).  Here's a sample command and sample output.

seekbnch a

Floppy Disk Seek Benchmark, Ver 1.0 by luitje@m-net.arbornet.org

Drive A: has 80 tracks; current Step Rate is 14

Track to track seek time:   3.0 mSecs

Average seek time:         81.7 mSecs

Full Stroke seek time:    309.5 mSecs

�����������������������������������������������������������������Ŀ

���� FLOPSKEW �����������������������������������������������������

�������������������������������������������������������������������

The most important improvement you can make in using your  floppy

drives  is  to  format  your disks using  FDFORMAT  by  Christoph

Hochst�tter or  ATFMT by Oleg Kibirev.   They can be found on the

Internet   in  any  of  the  SimTel  archives  in  the  directory

msdos/diskutil. These programs allow almost complete control over

how your disks are formatted.  Among other things, you can put up

to 820K on 5�" DSDD disks, 1.48 meg on 5�"  HD disks and 1.7  meg

on 3�" HD disks.   In terms of speeding up floppy disk transfers,

these programs allow you  to control the way sectors are arranged

on  a track.   This concept is called  sector sliding by FDFORMAT

and  sector  spinning by ATFMT;  in the US it's known  as  sector

skewing.    Basically,   the  idea is that since  there  is  some

mechanical delay in changing heads  or tracks it may be desirable

to put the first sector of a new  head or track  "later"   in the

track so the computer doesn't have to wait for the floppy to make

a complete revolution before that sector can be read.   See these

programs' documentation for more details on how this works.   The

time savings when  reading or writing a floppy can be substantial

when you format it using  the optimal skew parameters and,  as an

added benefit, formatting is faster, too.

You  can  specify head skew and track skew in both  FDFORMAT  and

ATFMT by the X and Y parameters,  respectively.   The DOS  FORMAT

program  does  not  support  sector  skewing  at  all;   this  is

equivalent  to  using FDFORMAT or ATFMT with X and Y both set  to

zero.    But  what  values should you use for  these  parameters?

Well,   when FDFORMAT first introduced this feature,  I  wrote  a

batch file which formatted a test disk with various  combinations

of the head and sector skew parameters and then measured how long

it  took  to copy a large file to the floppy.   It took about  an

hour to run and tested 16 combinations of X and Y values for just

one  disk  format.    FLOPSKEW does the same job in  just  a  few

seconds but tests even more combinations of parameter values.

To  use FLOPSKEW,  first decide which setting of FLOPPARM to  use

and then run FLOPPARM with that parameter since the step rate and

head settle time will both affect skewing.   Then,  format a disk

of  the type and capacity for which you want to find the  optimal

skewing  parameters  with  the  X and Y  parameters  both  0  and

interleave 1.   Although FLOPSKEW is a non-destructive test,  you

should  use  a  freshly  formatted  disk  to  ensure  that  these

parameters  are  set  correctly,  otherwise the  test  will  give

strange  results.    If you don't specify X and Y,  FDFORMAT  and

ATFMT  default to the correct values except that they  both  must

use  an interleave of 2 for the 5�"  1.44  meg and 3�"  1.7   meg

formats  (and for this reason it doesn't make sense to use sector

skewing  with these formats).  DOS FORMAT always formats with  no

sector skewing and interleave 1.

Then, simply run FLOPSKEW with the drive letter of the floppy you

want to test as the only parameter.   Here is an example  command

and  its  results  for a double density (720K)  3�"  disk  on  my

system:

flopskew b

Find Optimal Head & Track Skew, Ver 1.0

9 Sectors/Track, 80 Tracks, 2 Sides

Head Skew Times (milliseconds):

X:       0   1   2   3   4   5   6   7   8

Time:   32  54  74  95 116 137 158 178 199

Track Skew Times (milliseconds):

Y:       0   1   2   3   4   5   6   7   8

Time:  232  53  74  95 116 137 158 178 199

Interleave:  1

Minimum skewing parameters:  X=0, Y=1

Recommended skewing parameters:  X=1, Y=2

First  you are shown the format of the test diskette,  then  some

test results and finally the recommended skewing parameters.   If

you  are interested in some technical details about the test  you

should read the next few paragraphs; otherwise you can skip ahead

to  the  next section.   Just use the recommended  parameters  to

format  your disks and expect your diskette throughput to  become

about 30% faster.

Two  tests  are  run to find the best values for  the  Head  Skew

parameter  (X)  and the Track Skew parameter (Y).  The head  skew

test  is run by reading the last sector of the first track  using

head 0 and then measuring how long,  in milliseconds, it takes to

read  the first sector of that same track with head 1.   (If  the

disk is formatted with only one side this test is not run.)   The

track  skew  test is run by reading the last sector of the  first

track  using  head 1 (if testing a double sided disk)   and  then

measuring  how  long  it takes to read the first  sector  of  the

second track using head 0.  In the results above, the second line

tells  how  long this took,  in milliseconds,  for  each  of  the

sectors on the second track read.

Let's look at the example data to see what we can learn.   First,

notice  that the times to read different sectors in the head skew

test increase by about 22 milliseconds.  This is just the time it

takes to read one sector for this format.  The formula is

sector read time(ms) = 1000 * 60sec/min * rpm / sectors

For  this  format the drive rotates at 300  rpm and there  are  9

sectors  so  the  time  to  read  a  sector  works  out  to  22.2

milliseconds.  This formula can be modified to figure the time to

read  an entire track by dropping the final division by  sectors.

In this case, it is 200 ms.

To  determine the best values for X and Y,  the program finds the

smallest  time  and the corresponding number of  sectors  skipped

from the line above.  The data show that for my system no sectors

should  be  skipped when changing heads so the optimal head  skew

parameter,  X, is 0.  Typical values are 0 or 1 for most systems.

Now  look at the results for the track skewing test;  the time to

change tracks without skipping any sectors is 232!   This is  the

time  it  takes to read a whole track plus the sector  plus  some

overhead.   In this case the 0 sector skew provided by DOS format

and  the  default values used by FDFORMAT and ATFMT yield  a  big

waste  of  time when reading or writing multiple tracks.   On  my

system the optimal track skew parameter, Y, is 1.  Typically,  it

ranges from 1 to 3 on the systems I have looked at.

Now  notice  that  the program recommends  values  for  the  skew

parameters which are 1 greater than the optimal values.   This is

because the penalty for choosing a skew value which is too  small

is severe:  you have to wait a whole revolution, in this case 200

ms,   when changing tracks.   On the other hand,  the penalty for

having a skew value which is one too big is only 22  ms.    Since

some  software  may introduce an additional delay  when  changing

tracks,   it  is a safe bet to be conservative when choosing  the

skew values.   If you regularly use floppies on two computers you

should  run FLOPSKEW on both of them and choose the higher of the

recommended skew parameters.

Take  the values of X and Y found by the above procedure and  use

them to format your floppies in the future.   To make this  easy,

you  can create a batch file,  4DOS or CED alias,  or FDFORMAT or

ATFMT  configuration file with those parameters in it.   Any time

you format a floppy or read or write a floppy formatted this  way

it will go faster.

How much faster?   Well,  assuming the most likely case that only

track  seek is a problem and that the sector skewing is only  off

by  1 or 2,  then on average the computer has to wait for a  full

disk revolution every second track it reads.   Eliminate the wait

and  you  can reduce the time spent reading and writing  by  1/3.

When  formatting  a  diskette,  skewing only affects  the  verify

operation so the time reduction is somewhat less, about 20%.

�����������������������������������������������������������������Ŀ

���� FLOPRPM ������������������������������������������������������

�������������������������������������������������������������������

If   you  are  having  trouble  reading  or  writing   diskettes,

especially with a large number of sectors per track,  the problem

may  lie with the speed at which the drive spins.   FLOPRPM  will

test  this condition for you.   Simply place a formatted disk  in

the  drive you want to test and run FLOPRPM with the drive letter

as the only argument.  The program will then test and display the

rotation rate continuously until you press a key.   Normally, all

PC floppy drives will run within  an RPM or two of 300 except for

5�"  HD drives which run at 360.   If you are having problems and

the  speed  of your drive is off by more  than that or the  speed

varies  a  lot,   you may  be able  to get  better  operation  by

adjusting the speed  of your drive.   Unfortunately,  no  general

guidelines  can  be  given  about how to do that  because  drives

differ in their design.

�����������������������������������������������������������������Ŀ

���� FLOPINFO �����������������������������������������������������

�������������������������������������������������������������������

This program  simply  reads the formatting information of  a disk

and displays it.   With all of the different formats supported by

FDFORMAT  and  ATFMT it's nice to know what format  a  particular

disk  is.    Also,  since some of these formats require a TSR  to

read,   you might want to know if a particular floppy can be read

on  another  system  which  doesn't  have  a  copy  of  that  TSR

available.   Or,  perhaps you want to quickly find out whether  a

floppy is HD or DD.

FLOPINFO  takes  one parameter,  the drive letter.   Here  is  an

example command and response, this time for a 1.4M 3�" disk.

flopinfo b

�������������������������������������������������������������������������ͻ

�     Floppy Disk Information, Ver. 1.1 by luitje@m-net.arbornet.org      �

�������������������������������������������������������������������������͹

� Sides � Tracks � Sec/Track � Bytes/Sec � Res/Hid Sec � Root Dir � Media �

�������������������������������������������������������������������������Ķ

�   2   �   80   �    18     �    512    �    1 / 0    �   224    �   F0  �

�������������������������������������������������������������������������͹

�  OEM Name  �  Volume Name  � Vol Ser Num �  FAT Type  � FATs � Capacity �

�������������������������������������������������������������������������Ķ

� "MSDOS5.0" � "NO NAME    " �  04DA-F7FE  � "FAT12   " �   2  �  1457664 �

�������������������������������������������������������������������������ͼ

The  Volume  Name displayed is from the boot  sector;   there  is

another  one in the root directory which may be different.    The

Volume Name, Volume Serial Number and FAT Type are not present on

disks  formated  with  versions  of DOS prior  to  4.0   and  are

displayed as dots in that case.  The "Res/Hid Sec"  box shows two

numbers,  the number of reserved and hidden sectors.   "Root Dir"

shows the number of files allowed in the root directory.  "Media"

is the media descriptor byte.

�����������������������������������������������������������������Ŀ

���� FLOPSCRB �����������������������������������������������������

�������������������������������������������������������������������

If you've ever bought one of those inexpensive disk cleaning kits

you've  probably  noticed that the instructions which  come  with

them are kind of skimpy.   Something like:  "squirt some cleaning

fluid  on  the  cleaning disk and then operate the drive  for  10

seconds". But how do you "operate the drive"?

You  could  use the dir command but it will time out with a  read

error  after  a  few  seconds so  you  have  to  keep  re-issuing

commands.   Even worse,  the dir command only tries to read track

zero  so  you  are  only using a small portion  of  the  cleaning

surface.  Or you could use other programs which access the  disk,

like format, but they all have the same problems:  it's difficult

to  control how long the cleaning is done and which part  of  the

cleaning disk is used.

FLOPSCRB  takes care of these problems and in addition  may allow

the cleaning disk to be used longer because of the way it manages

the cleaning process.  FLOPSCRB cleans the drive in three stages.

The  first stage uses the outside third (which  has  the  largest

area)   of  the  cleaning disk.   This removes the worst  of  the

accumulated oxide.  The second stage moves to the middle third of

the  disk  where any remaining particles are moved.    The  third

stage moves to the inner third of the cleaning disk to finish the

job.   With this sequence the head becomes progressively  cleaner

instead  of  just  rubbing  dirt off on  the  cleaning  disk  one

revolution and then picking it up the next.

FLOPSCRB only needs to be told which drive to clean (only A and B

are  supported  at this time)  and how long to run  the  cleaning

action.    To clean drive a for 15  seconds (5  seconds for  each

stage) you'd use the following command:

flopscrb a 15

NOTE:  The abrasiveness of the cleaning pads may vary from kit to

kit  so  always  follow  the  manufacturer's  instructions  about

cleaning time. It may be possible to damage the floppy drive head

by cleaning the drive excessively.

�����������������������������������������������������������������Ŀ

���� Acknowledgements ���������������������������������������������

�������������������������������������������������������������������

Outside  of the cryptic documentation for the NEC �PD765A  floppy

disk  controller  chip there is not a whole  lot  of  information

available  about the nitty gritty of controlling floppies.    I'd

like  to thank two freeware authors who have  written  worthwhile

programs  and then published the source code for others to  learn

from.   Christoph  Hochst�tter's FDFORMAT provides  lots  of good

examples of code for using the BIOS INT 13 calls and creating the

basic  disk  structure (it also lets you learn  a  little  German

along the way).  The 2M package by Ciriaco de Celis provides some

utilities  which really push the envelope of putting the  maximum

amount  of  information  on a floppy disk.   I  needed  to  issue

commands directly to the floppy drive controller to make FLOPSCRB

work and I learned how to  do  this (and some Spanish) by reading

his source code.   Understanding these programs can be slow going

if  you  don't  understand German and Spanish but it  sure  beats

disassembling  your  BIOS!  Look for them in  the  msdos/diskutil

directory of your local SimTel mirror.

�����������������������������������������������������������������Ŀ

���� Distribution �������������������������������������������������

�������������������������������������������������������������������

The programs and documentation in the Floppy Tool Kit package are

copyrighted  works  of William Luitje.   The author grants you  a

non-exclusive license to copy and distribute these  works as long

as they are distributed as a group with no additional files added

to  the  archive  (this includes BBS ads)  and the  programs  and

documentation are not modified in any way.

On a related note, I am distributing these programs for free.  If

you paid money for a floppy or CD-ROM with these programs on  it,

be  aware that I am not seeing a penny of it.   What you paid for

is the service of distribution.  I'm sure that you understand the

nature of transactions like that but I have heard from people who

"bought  one  of  my  programs"   and  expect  me  to  conduct  a

correspondence course on basic PC  techniques,  theory and repair

because  of  their  "purchase".   While I am always open  to  bug

reports and well  thought out questions (and postcards!),  please

read  the  documentation carefully and check with  local  experts

before you fire up your word processor.

�����������������������������������������������������������������Ŀ

���� Liability ����������������������������������������������������

�������������������������������������������������������������������

This  software  is intended to be safe to use.    All  tests  and

information  programs are read only so no data on your test disks

should be altered. FLOPSCRB and SEEKBNCH could conceivably damage

a drive if used inappropriately but I abused  my  drives quite  a

bit while writing and  debugging  these  programs and haven't had

any  problems.    About the only other problem you might have  is

that if you set the floppy drive parameters  too fast you may get

read or write errors.  Never the less, in using this software you

assume all risk should anything go wrong.  I think that is pretty

unlikely, but, hey, sometimes it rains fish!

�����������������������������������������������������������������Ŀ

���� Registration �������������������������������������������������

�������������������������������������������������������������������

While this software may be freely copied and used,  I  do request

that  you  register  it if you use it.   Just send  me  either  a

postcard or some e-mail telling me which programs you find useful

and  which version of the package you are using.   This  is  only

partially an ego massage for the author.   I enjoy writing useful

utilities and have several other pieces of freeware out  floating

around  the  net so I would like feedback about which of them are

worth expanding and supporting.

William Luitje

2677 Wayside Drive

Ann Arbor, MI 48103

USA

Internet:  luitje@m-net.arbornet.org

�����������������������������������������������������������������Ŀ

���� History ������������������������������������������������������

�������������������������������������������������������������������

Version 1.0 20 Jan 94

First  release with FLOPPARM, FLOPTEST, FLOPSKEW,  FLOPRPM and

FLOPINFO

Version 1.1 13 Mar 95

Changes to FLOPPARM:  tweaked FAST parameters, changed

allowable ranges of parameters

Changes to FLOPINFO:  added volume name, volume serial number,

FAT type, hidden sectors and media byte to output.  Reorganized

display for readability.

Minor internal changes to the rest of the programs.

Added TESTSTEP

Added SEEKBNCH

Added FLOPSCRB

Copyright 1994-1995 by William Luitje