CALLING ASMLIB FROM C PROGRAMS

I have used many ASMLIB subroutines with Turbo C and Borland C++.  The

following comments and helpful hints apply to both compilers even if I

only mention Turbo C.

You can call most ASMLIB subroutines from Turbo C if you follow these

simple rules:

�)  Tell your C program that the ASMLIB subroutine is a PASCAL function

This makes the C compiler convert the subroutine name to upper case

and supresses the _leading _underscore in the subroutine name.  It's

good practice to declare the subroutine in UPPER CASE in the event of

name conflicts (such as STRLEN).

/*  Example:  */

extern  void pascal TPRINT(void);

�)  Load the 80x86 registers before calling the ASMLIB subroutine.  Be

sure to save the DI and SI registers because Turbo C expects that

these registers will not change across subroutien calls.

/*  Example:  */

asm  push si;          /* TC wants SI to be saved */

_DX = 0;               /* upper left corner of screen */

_AH = 15;              /* bright white */

_SI = msg;             /* pointer to ASCIIZ string */

tprint();

asm  pop  si;          /* restore SI register */

�)  ASMLIB subroutines using the 8087 will use Turbo C's emulation

library if re-assembled with the /E option (except TINY model).

�)  values in registers returned by ASMLIB subroutines should be used

or saved immediately; C code executed after calling an ASMLIB

subroutine may change any of the registers.

�)  When compiling from within the Turbo C editor, include ASM86M.LIB

in your .PRJ file (ASM86S.LIB for small model, ASM86H.LIB for

huge model).

�)  ASMLIB's HUGE model library does not strictly adhere to Turbo C's HUGE

model assumptions.  Many ASMLIB HUGE model subroutines assume DS:DGROUP,

while the standard HUGE model assumes DS:nothing.  (ASMLIB also does

not provide a LARGE model library; the ASMLIB MEDIUM model libraries

may be used instead, with the same treatment of DS).

/*  Example:                                            */

/*  use ASMLIB's DRAWLINE subroutine; assumes DS:DGROUP */

extern void pascal DRAWLINE(void);

asm  push ds;           /* save DS segment - this may not be nessesary */

asm  mov  ax,ss;        /* get DROUP segment                           */

asm  mov  ds,ax;        /* DS = DGROUP                                 */

_BX=&x0;                /* point to line endpoint data IN LOCAL MEMORY */

DRAWLINE();

asm  pop  ds;           /* restore DS segment if pushed                */

�)  Many ASMLIB subroutines use the ASMLIB near heap during execution.

Subroutines in ASMLIB's H2MALLOC.ASM intercept ASMLIB subroutines'

calls to ASMLIB near heap functions and use the C compiler's

malloc, free and realloc library functions instad of ASMLIB's heap.

H2MALLOC must be assembled with the TASM or MASM /mx switch, and H2MALLOC

must be specified must be specified in your list of object modules.

MS-LINK also wants you to use the /NOE command-line switch.