******************************************************************************

VARIAN ASYNC32 COMPONENT v1.25

(c) VARIAN SOFTWARE SERVICES NL 1996-1997

ALL RIGHTS RESERVED

******************************************************************************

TCOMM EXCEPTIONS

================

type

ECommError = class(Exception)

ErrorCode: Integer;

end;

An ECommError exception contains an additional errorcode wich includes an

windows system error result. This value is set to -1 when the error is

handled by the component itself and no extra errorcode is available.

PUBLISHED PROPERTIES

====================

DeviceName

property DeviceName: string;

Name of communication device, Com1, Com2, Lpt1 etc. Can only be

set when the port is not assigned (closed).

MonitorEvents

TCommEventState = (evBreak, evCTS, evDSR, evError, evRing,

evRlsd, evRxChar, evRxFlag, evTxEmpty);

TCommEventType = set of TCommEventState;

property MonitorEvents: TCommEventType;

Enable or disable certain system events.

evBREAK   - A break was detected on input.

evCTS     - The CTS (clear-to-send) signal changed state.

evDSR     - The DSR (data-set-ready) signal changed state.

evERROR   - A line-status error occurred. Line-status errors

are CE_FRAME, CE_OVERRUN, and CE_RXPARITY.

evRING    - A ring indicator was detected.

evRLSD    - The RLSD (receive-line-signal-detect) signal changed state.

evRXCHAR  - A character was received and placed in the input buffer.

evRXFLAG  - The event character was received and placed in the input buffer.

The event character is specified in the device's DCB structure,

which is applied to a serial port by using the SetCommState function.

evTXEMPTY - The last character in the output buffer was sent.

Baudrate

TBaudRate = (cbr110, cbr300, cbr600, cbr1200, cbr2400, cbr4800, cbr9600,

cbr14400, cbr19200, cbr38400, cbr56000, cbr57600, cbr115200, cbr128000, cbr256000);

property BaudRate: TBaudRate;

Parity

TParity = (paNone, paOdd, paEven, paMark, paSpace);

property Parity: TParity;

Stopbits

TStopbits = (sb10, sb15, sb20);

property Stopbits: TStopbits;

Databits

TDatabits=(da4, da5, da6, da7, da8);

property Databits: TDatabits;

ReadBufSize

property ReadBufSize: Integer;

Specifies the recommended size, in bytes, of the device's internal input buffer.

WriteBufSize

property WriteBufSize: Integer;

Specifies the recommended size, in bytes, of the device's internal output buffer.

CharsTimeout

property CharsTimeout: Integer;

Specifies the maximum time, in milliseconds, allowed to elapse between

the arrival of two characters on the communications line. During a Read

operation, the time period begins when the first character is received.

If the interval between the arrival of any two characters exceeds this

amount, the Read operation is completed and any buffered data is returned.

A value of zero indicates that interval time-outs are not used.

Options

TCommOption = (coParityCheck, coDsrSensitivity, coIgnoreXOff,

coErrorChar, coNullStrip);

TCommOptions = set of TCommOption;

property Options: TCommOptions;

coParityCheck:

Specifies whether parity checking is enabled. If this member is TRUE,

parity checking is performed and errors are reported.

coDsrSensitivity:

Specifies whether the communications driver is sensitive to the state

of the DSR signal. If this member is TRUE, the driver ignores any bytes

received, unless the DSR modem input line is high.

coIgnoreXoff:

Specifies whether transmission stops when the input buffer is full and the

driver has transmitted the XoffChar character. If this member is TRUE,

transmission continues after the input buffer has come within XoffLim bytes

of being full and the driver has transmitted the XoffChar character to stop

receiving bytes. If this member is FALSE, transmission does not continue until

the input buffer is within XonLim bytes of being empty and the driver has

transmitted the XonChar character to resume reception.

coErrorChar:

Specifies whether bytes received with parity errors are replaced with the

character specified by the ErrorChar member. If this member is TRUE and

the fParity member is TRUE, replacement occurs.

coNullStrip:

Specifies whether null bytes are discarded. If this member is TRUE,

null bytes are discarded when received.

EventChars

XonChar

property XonChar: char;

Specifies the value of the XON character for both transmission and reception

XoffChar

property XoffChar: char;

Specifies the value of the XOFF character for both transmission and reception.

FlowControl

TFlowControl = (fcNone, fcCTS, fcDTR, fcSoftware, fcDefault);

property FlowControl: TFlowControl;

fcNone                No flowcontrol

fsCTS                 Rts/Cts flowcontrol

fsDTR                 Specifies the DTR (data-terminal-ready) flow control.

fsSoftware            Xon/Xoff flowcontrol

fcDefault             Use system settings

METHODS AND PUBLIC PROPERTIES

=============================

Enabled

function Enabled: Boolean;

Describes the component/device state, true=Open, false=Closed

Open

procedure Open;

Opens the communications device and does all the needed configurations.

Open must be called in order to access the comport. If an invalid

device handle is requested, one of the following errorcodes is returned.

2      File not found.

3      Invalid file name.

4      Too many open files.

5      Access denied.

100    EOF.

101    Disk full.

106    Invalid input.

Close

procedure Close;

Close the communications device.

Write

function Write(const Buf; Count: Integer): Integer;

Buf is any variable, Count is an expression of type Integer. Write writes Count

or fewer bytes to the com port from memory, starting at the first byte occupied

by Buf. The actual number of bytes written (less than or equal to Count) is

returned in it's Result. If a write operations fails it returns -1.

Read

function Read(var Buf; Count: Integer): Integer;

Buf is any variable, Count is an expression of type Integer. Read reades Count

or fewer bytes from the com port. The actual number of bytes read

(less than or equal to Count) is returned in it's Result. If a read operations

fails it returns -1. Never try to read bytes manually by specifying the

count parameter yourself. Use the Count parameter returned in "RxCharEvent".

InQueCount

function InQueCount: Integer;

Number of bytes in the ReadBuffer.

OutQueCount

function OutQueCount: Integer;

Number of bytes in the WriteBuffer. When all bytes are sent, an OnTxEmpty

event is triggerd.

PurgeIn

procedure PurgeIn;

Remove all pending characters from the inputbuffer (read).

PurgeOut

procedure PurgeOut;

Remove all pending characters from the outputbuffer (write).

SetDTRState

procedure SetDTRState(State: Boolean);

False - Clears the DTR (data-terminal-ready) signal.

True  - Sends the DTR (data-terminal-ready) signal.

SetRTSState

procedure SetRTSState(State: Boolean);

False - Clears the RTS (request-to-send) signal.

True  - Sends the RTS (request-to-send) signal.

SetBreakState

procedure SetBREAKState(State: Boolean);

False - Restores character transmission and places the transmission line

in a nonbreak state.

True  - Suspends character transmission and places the transmission line in

a break state. Note that this extended function does not flush data

that has not been transmitted.

SetXONState

procedure SetXONState(State: Boolean);

False - Causes transmission to act as if an XOFF character has been received.

True  - Causes transmission to act as if an XON character has been received.

CTS

property CTS: Boolean;

True - The CTS (clear-to-send) signal is on.

DSR

property DSR: Boolean;

True - The DSR (data-set-ready) signal is on.

RING

property RING: Boolean;

True - The ring indicator signal is on.

RLSD

property RLSD: Boolean;

True - The RLSD (receive-line-signal-detect) signal is on.

Handle

property Handle: THandle;

Reference to the internal devicehandle (Readonly property)

ProviderSubtype

property ProviderSubtype: Integer;

Device Id. Use the added GetProviderSubTypeName function in order

to get a string representation of the type of device currently

opened.

Available provider subtypes:

PST_FAX            FAX device

PST_LAT            LAT protocol

PST_MODEM          Modem device

PST_NETWORK_BRIDGE Unspecified network bridge

PST_PARALLELPORT   Parallel port

PST_RS232          RS-232 serial port

PST_RS422          RS-422 port

PST_RS423          RS-423 port

PST_RS449          RS-449 port

PST_SCANNER        Scanner device

PST_TCPIP_TELNET   TCP/IP Telnet® protocol

PST_UNSPECIFIED    Unspecified

PST_X25            X.25 standards

EVENTS

======

OnBreak

property OnBreak: TNotifyEvent;

A break was detected on input.

OnCTS

property OnCTS: TNotifyEvent;

The CTS (clear-to-send) signal changed state.

OnDSR

property OnDSR: TNotifyEvent;

The DSR (data-set-ready) signal changed state.

OnRing

property OnRing: TNotifyEvent;

A ring indicator was detected.

OnRLSD

property OnRLSD: TNotifyEvent;

The RLSD (receive-line-signal-detect) signal changed state.

OnError

TCommErrorEvent = procedure(Sender: TObject; Errors: Integer) of object;

property OnError: TCommErrorEvent;

A line-status error occurred.

CE_BREAK       The hardware detected a break condition.

CE_DNS         Windows 95 only: A parallel device is not selected.

CE_FRAME       The hardware detected a framing error.

CE_IOE         An I/O error occurred during communications with the device.

CE_MODE        The requested mode is not supported, or the hFile parameter

is invalid. If this value is specified, it is the only valid error.

CE_OOP         Windows 95 only: A parallel device signaled that it is out of paper.

CE_OVERRUN     A character-buffer overrun has occurred. The next character is lost.

CE_PTO         Windows 95 only: A time-out occurred on a parallel device.

CE_RXOVER      An input buffer overflow has occurred. There is either no

room in the input buffer, or a character was received after

the end-of-file (EOF) character.

CE_RXPARITY    The hardware detected a parity error.

CE_TXFULL      The application tried to transmit a character, but the output

buffer was full.

OnRxChar

TCommRxCharEvent = procedure(Sender: TObject; Count: Integer) of object;

property OnRxChar: TCommRxCharEvent;

A character was received and placed in the input buffer. Count defines

the number of bytes received.

OnRxFlag

property OnRxFlag: TNotifyEvent;

The event character was received and placed in the input buffer.

The event character is specified in the device's DCB structure,

which is applied to a serial port by using the SetCommState function.

OnTxEmpty

property OnTxEmpty: TNotifyEvent;

The last character in the output buffer was sent.