!! Version 3.0

!!

!! Deutscher Text siehe scc_ger.doc (Vorsicht: veraltet!)

!!

!! Please note that the information in this document may be hopelessly

!! outdated. A newer version of the documentation may be available on

!! http://qsl.net/dl1bke/

!!

!! BTW: REAL programmers don't document...

!!

SCC.C - Linux driver for Z8530 based HDLC cards for AX.25

********************************************************************

Copyright 1993-1998 by Joerg Reuter DL1BKE

portions (c) 1993 Guido ten Dolle PE1NNZ

for the complete copyright notice see >> Copying.Z8530DRV <<

********************************************************************

0. Quickstart

=============

To compile the utilities, type

make dep ; make

sccinit  - reads /etc/ax25/z8530drv.conf and initializes the driver

sccstat  - shows the status of a channel

sccparam - sets KISS parameters for a channel

It will install these utilities in /sbin/ and an example

configuration file as /etc/ax25/z8530drv.conf (z8530drv.conf.ex

if the file already exists).

Please note that since version 3.0 of this driver the driver

itself does not come with this package (hence the package name

"z8530drv-*.*-utils.tar.gz").

=====

Now configure the Linux kernel to prepare the compilation of

the driver. If you are using menuconfig or xconfig select:

Network device support

Radio network interfaces (CONFIG_NET_RADIO): "Y"

Z8530 SCC driver for AX.25 (CONFIG_SCC): "Y" or "M"

Do not forget to select other AX.25 devices you probably need,

and do not forget to include the kernel AX.25 / NET/ROM / Rose

stuff as well. Please read the AX.25-HOWTO for details.

Save your new kernel configuration and either rebuild your kernel

now (if you selected "Y" for CONFIG_SCC), or rebuild your modules

(if you selected "M").

1. Initialization of the driver

===============================

To use the driver, 3 steps must be performed:

1. if compiled as module: loading the module

2. Setup of hardware, MODEM and KISS parameters with sccinit

3. Attach each channel to the Linux kernel AX.25 with "ifconfig"

Unlike the versions below 2.4 this driver is a real network device

driver. If you want to run xNOS instead of our fine kernel AX.25

use a 2.x version (available from above sites) or read the

AX.25-HOWTO on how to emulate a KISS TNC on network device drivers.

1.1 Loading the module

======================

(If you're going to compile the driver as a part of the kernel image,

skip this chapter and continue with 1.2)

Before you can use a module, you'll have to load it with

insmod scc

please read 'man insmod' that comes with modutils.

You should include the insmod in one of the /etc/rc.d/rc.* files,

and don't forget to insert a call of sccinit after that. It

will read your /etc/ax25/z8530drv.conf.

1.2. /etc/ax25/z8530drv.conf

============================

To setup all parameters you must run /sbin/sccinit from one

of your rc.*-files. This has to be done BEFORE you can

"ifconfig" an interface. Sccinit reads the file /etc/ax25/z8530drv.conf

and sets the hardware, MODEM and KISS parameters. A sample file is

delivered with this package. Change it to your needs.

The file itself consists of two main sections.

1.2.1 configuration of hardware parameters

==========================================

The hardware setup section defines the following parameters for each

Z8530:

chip    1

data_a  0x300                   # data port A

ctrl_a  0x304                   # control port A

data_b  0x301                   # data port B

ctrl_b  0x305                   # control port B

irq     5                       # IRQ No. 5

pclock  4915200                 # clock

board   BAYCOM                  # hardware type

escc    no                      # enhanced SCC chip? (8580/85180/85280)

vector  0                       # latch for interrupt vector

special no                      # address of special function register

option  0                       # option to set via sfr

chip	- this is just a delimiter to make sccinit a bit simpler to

program. A parameter has no effect.

data_a  - the address of the data port A of this Z8530 (needed)

ctrl_a  - the address of the control port A (needed)

data_b  - the address of the data port B (needed)

ctrl_b  - the address of the control port B (needed)

irq     - the used IRQ for this chip. Different chips can use different

IRQs or the same. If they share an interrupt, it needs to be

specified within one chip-definition only.

pclock  - the clock at the PCLK pin of the Z8530 (option, 4915200 is

default), measured in Hertz

board   - the "type" of the board:

SCC type                 value

---------------------------------

PA0HZP SCC card          PA0HZP

EAGLE card               EAGLE

PC100 card               PC100

PRIMUS-PC (DG9BL) card   PRIMUS

BayCom (U)SCC card       BAYCOM

escc    - if you want support for ESCC chips (8580, 85180, 85280), set

this to "yes" (option, defaults to "no")

vector  - address of the vector latch (aka "intack port") for PA0HZP

cards. There can be only one vector latch for all chips!

(option, defaults to 0)

special - address of the special function register on several cards.

(option, defaults to 0)

option  - The value you write into that register (option, default is 0)

You can specify up to four chips (8 channels). If this is not enough,

just change

#define MAXSCC 4

to a higher value.

Example for the BayCom USCC:

----------------------------

chip    1

data_a  0x300                   # data port A

ctrl_a  0x304                   # control port A

data_b  0x301                   # data port B

ctrl_b  0x305                   # control port B

irq     5                       # IRQ No. 5 (#)

board   BAYCOM                  # hardware type (*)

#

# SCC chip 2

#

chip    2

data_a  0x302

ctrl_a  0x306

data_b  0x303

ctrl_b  0x307

board   BAYCOM

An example for a PA0HZP card:

-----------------------------

chip 1

data_a 0x153

data_b 0x151

ctrl_a 0x152

ctrl_b 0x150

irq 9

pclock 4915200

board PA0HZP

vector 0x168

escc no

#

#

#

chip 2

data_a 0x157

data_b 0x155

ctrl_a 0x156

ctrl_b 0x154

irq 9

pclock 4915200

board PA0HZP

vector 0x168

escc no

A DRSI would should probably work with this:

--------------------------------------------

(actually: two DRSI cards...)

chip 1

data_a 0x303

data_b 0x301

ctrl_a 0x302

ctrl_b 0x300

irq 7

pclock 4915200

board DRSI

escc no

#

#

#

chip 2

data_a 0x313

data_b 0x311

ctrl_a 0x312

ctrl_b 0x310

irq 7

pclock 4915200

board DRSI

escc no

Note that you cannot use the on-board baudrate generator off DRSI

cards. Use "mode dpll" for clock source (see below).

This is based on information provided by Mike Bilow (and verified

by Paul Helay)

The utility "sccgencfg"

-----------------------

If you only know the parameters for the PE1CHL driver for DOS,

run sccgencfg. It will generate the correct port addresses (I hope).

Its parameters are exactly the same as the ones you use with

the "attach scc" command in net, except that the string "init" must

not appear. Example:

sccgencfg 2 0x150 4 2 0 1 0x168 9 4915200

will print a skeleton z8530drv.conf for the OptoSCC to stdout.

sccgencfg 2 0x300 2 4 5 -4 0 7 4915200 0x10

does the same for the BayCom USCC card. I my opinion it is much easier

to edit scc_config.h...

1.2.2 channel configuration

===========================

The channel definition is divided into three sub sections for each

channel:

An example for scc0:

# DEVICE

device scc0	# the device for the following params

# MODEM / BUFFERS

speed 1200		# the default baudrate

clock dpll		# clock source:

# 	dpll     = normal halfduplex operation

# 	external = MODEM provides own Rx/Tx clock

#	divider  = use fullduplex divider if

#		   installed (1)

mode nrzi		# HDLC encoding mode

#	nrzi = 1k2 MODEM, G3RUH 9k6 MODEM

#	nrz  = DF9IC 9k6 MODEM

#

bufsize	384		# size of buffers. Note that this must include

# the AX.25 header, not only the data field!

# (optional, defaults to 384)

# KISS (Layer 1)

txdelay 36              # (see chapter 1.4)

persist 64

slot    8

tail    8

fulldup 0

wait    12

min     3

maxkey  7

idle    3

maxdef  120

group   0

txoff   off

softdcd on

slip    off

The order WITHIN these sections is unimportant. The order OF these

sections IS important. The MODEM parameters are set with the first

recognized KISS parameter...

Please note that you can initialize the board only once after boot

(or insmod). You can change all parameters but "mode" and "clock"

later with the Sccparam program or through KISS. Just to avoid

security holes...

(1) this divider is usually mounted on the SCC-PBC (PA0HZP) or not

present at all (BayCom). It feeds back the output of the DPLL

(digital pll) as transmit clock. Using this mode without a divider

installed will normally result in keying the transceiver until

maxkey expires --- of course without sending anything (useful).

2. Attachment of a channel by your AX.25 software

=================================================

2.1 Kernel AX.25

================

To set up an AX.25 device you can simply type:

ifconfig scc0 44.128.1.1 hw ax25 dl0tha-7

This will create a network interface with the IP number 44.128.20.107

and the callsign "dl0tha". If you do not have any IP number (yet) you

can use any of the 44.128.0.0 network. Note that you do not need

axattach. The purpose of axattach (like slattach) is to create a KISS

network device linked to a TTY. Please read the documentation of the

ax25-utils and the AX.25-HOWTO to learn how to set the parameters of

the kernel AX.25.

2.2 NOS, NET and TFKISS

=======================

Since the TTY driver (aka KISS TNC emulation) is gone you need

to emulate the old behaviour. The cost using these programs is

that you probably need to compile the kernel AX.25, regardless

if you actually use it or not. First setup your /etc/ax25/axports,

for example:

9k6	dl0tha-9  9600  255 4 9600 baud port (scc3)

axlink	dl0tha-15 38400 255 4 Link to NOS

Now "ifconfig" the scc device:

ifconfig scc3 44.128.1.1 hw ax25 dl0tha-9

You can now axattach a pseudo-TTY:

axattach /dev/ptys0 axlink

and start your NOS and attach /dev/ptys0 there. The problem is that

NOS is reachable only via digipeating through the kernel AX.25

(disasterous on a DAMA controlled channel). To solve this problem,

configure "rxecho" to echo the incoming frames from "9k6" to "axlink"

and outgoing frames from "axlink" to "9k6" and start:

rxecho

Or simply use "kissbridge" coming with this package:

ifconfig scc3 hw ax25 dl0tha-9

kissbridge scc3 /dev/ptys0

There's a similar program called net2kiss coming with the

AX.25 utilities which does basicly the same (but the

source looks better than mine...) and is better documented.

3. Adjustment and Display of parameters

=======================================

3.1 Displaying SCC Parameters:

==============================

Once a SCC channel has been attached, the parameter settings and

some statistic information can be shown using the param program:

dl1bke-u:~$ sccstat scc0

Parameters:

speed       : 1200 baud

txdelay     : 36

persist     : 255

slottime    : 0

txtail      : 8

fulldup     : 1

waittime    : 12

mintime     : 3 sec

maxkeyup    : 7 sec

idletime    : 3 sec

maxdefer    : 120 sec

group       : 0x00

txoff       : off

softdcd     : on

SLIP        : off

Status:

HDLC                  Z8530           Interrupts         Buffers

-----------------------------------------------------------------------

Sent       :     273  RxOver :     0  RxInts :   125074  Size    :  384

Received   :    1095  TxUnder:     0  TxInts :     4684  NoSpace :    0

RxErrors   :    1591                  ExInts :    11776

TxErrors   :       0                  SpInts :     1503

Tx State   :    idle

The status info shown is:

Sent		- number of frames transmitted

Received	- number of frames received

RxErrors	- number of receive errors (CRC, ABORT)

TxErrors	- number of discarded Tx frames (due to various reasons)

Tx State	- status of the Tx interrupt handler: idle/busy/active/tail (2)

RxOver		- number of receiver overruns

TxUnder		- number of transmitter underruns

RxInts		- number of receiver interrupts

TxInts		- number of transmitter interrupts

EpInts		- number of receiver special condition interrupts

SpInts		- number of external/status interrupts

Size		- maximum size of an AX.25 frame (*with* AX.25 headers!)

NoSpace		- number of times a buffer could not get allocated

An overrun is abnormal. If lots of these occur, the product of

baudrate and number of interfaces is too high for the processing

power of you computer. NoSpace errors are not likely caused by the

driver or the kernel AX.25.

3.2 Setting Parameters

======================

The setting of the channel access parameters is done in the  same way

as a KISS TNC. You can change parameters by using the kissparms program

from the ax25-utils package or use the program "sccparam":

sccparam <device> <paramname> <decimal-|hexadecimal value>

You can change the following parameters:

param	    : value

------------------------

speed       : 1200

txdelay     : 36

persist     : 255

slottime    : 0

txtail      : 8

fulldup     : 1

waittime    : 12

mintime     : 3

maxkeyup    : 7

idletime    : 3

maxdefer    : 120

group       : 0x00

txoff       : off

softdcd     : on

SLIP        : off

The parameters have the following meaning:

speed:

The baudrate on this channel in bits/sec

Example: sccparam scc3 speed 9600

txdelay:

The delay (in units of 10ms) after keying of the

transmitter, until the first byte is sent. This is usually

called "TXDELAY" in a TNC.  When 0 is specified, the driver

will just wait until the CTS signal is asserted. This

assumes the presence of a timer or other circuitry in the

MODEM and/or transmitter, that asserts CTS when the

transmitter is ready for data.

A normal value of this parameter is 30-36.

Example: sccparam scc0 txd 20

persist:

This is the probability that the transmitter will be keyed

when the channel is found to be free.  It is a value from 0

to 255, and the probability is (value+1)/256.  The value

should be somewhere near 50-60, and should be lowered when

the channel is used more heavily.

Example: sccparam scc2 persist 20

slottime:

This is the time between samples of the channel. It is

expressed in units of 10ms.  About 200-300 ms (value 20-30)

seems to be a good value.

Example: sccparam scc0 slot 20

tail:

The time the transmitter will remain keyed after the last

byte of a packet has been transferred to the SCC. This is

necessary because the CRC and a flag still have to leave the

SCC before the transmitter is keyed down. The value depends

on the baudrate selected.  A few character times should be

sufficient, e.g. 40ms at 1200 baud. (value 4)

The value of this parameter is in 10ms units.

Example: sccparam scc2 4

full:

The full-duplex mode switch. This can be one of the following

values:

0:   The interface will operate in CSMA mode (the normal

half-duplex packet radio operation)

1:   Fullduplex mode, i.e. the transmitter will be keyed at

any time, without checking the received carrier.  It

will be unkeyed when there are no packets to be sent.

2:   Like 1, but the transmitter will remain keyed, also

when there are no packets to be sent.  Flags will be

sent in that case, until a timeout (parameter 10)

occurs.

Example: sccparam scc0 fulldup off

wait:

The initial waittime before any transmit attempt, after the

frame has been queue for transmit.  This is the length of

the first slot in CSMA mode.  In fullduplex modes it is

set to 0 for maximum performance.

The value of this parameter is in 10ms units.

Example: sccparam scc1 wait 4

maxkey:

The maximal time the transmitter will be keyed to send

packets, in seconds.  This can be useful on busy CSMA

channels, to avoid "getting a bad reputation" when you are

generating a lot of traffic.  After the specified time has

elapsed, no new frame will be started. Instead, the trans-

mitter will be switched off for a specified time (parameter

min), and then the selected algorithm for keyup will be

started again.

The value 0 as well as "off" will disable this feature,

and allow infinite transmission time.

Example: sccparam scc0 maxk 20

min:

This is the time the transmitter will be switched off when

the maximum transmission time is exceeded.

Example: sccparam scc3 min 10

idle

This parameter specifies the maximum idle time in fullduplex

2 mode, in seconds.  When no frames have been sent for this

time, the transmitter will be keyed down.  A value of 0 is

has same result as the fullduplex mode 1. This parameter

can be disabled.

Example: sccparam scc2 idle off	# transmit forever

maxdefer

This is the maximum time (in seconds) to wait for a free channel

to send. When this timer expires the transmitter will be keyed

IMMEDIATELY. If you love to get trouble with other users you

should set this to a very low value ;-)

Example: sccparam scc0 maxdefer 240	# 2 minutes

txoff:

When this parameter has the value 0, the transmission of packets

is enable. Otherwise it is disabled.

Example: sccparam scc2 txoff on

group:

It is possible to build special radio equipment to use more than

one frequency on the same bad, e.g. using several receivers and

only one transmitter that can be switched between frequencies.

Also, you can connect several radios that are active on the same

band.  In these cases, it is not possible, or not a good idea, to

transmit on more than one frequency.  The SCC driver provides a

method to lock transmitters on different interfaces, using the

"param <interface> group <x>" command.  This will only work when

you are using CSMA mode (parameter full = 0).

The number <x> must be 0 if you want no group restrictions, and

can be computed as follows to create restricted groups:

<x> is the sum of some OCTAL numbers:

200  This transmitter will only be keyed when all other

transmitters in the group are off.

100  This transmitter will only be keyed when the carrier

detect of all other interfaces in the group is off.

0xx  A byte that can be used to define different groups.

Interfaces are in the same group, when the logical AND

between their xx values is nonzero.

Examples:

When 2 interfaces use group 201, their transmitters will never be

keyed at the same time.

When 2 interfaces use group 101, the transmitters will only key

when both channels are clear at the same time.  When group 301,

the transmitters will not be keyed at the same time.

Don't forget to convert the octal numbers into decimal before

you set the parameter.

Example: (to be written)

softdcd:

use a software dcd instead of the real one... Useful for a very

slow squelch.

Example: sccparam scc0 soft on

4. Problems

===========

If you have tx-problems with your BayCom USCC card please check

the manufacturer of the 8530. SGS chips have a slightly

different timing. Try Zilog...  A solution is to write to register 8

instead to the data port, but this won't work with the ESCC chips.

*SIGH!*

A very common problem is that the PTT locks until the maxkeyup timer

expires, although interrupts and clock source are correct. In most

cases #define SCC_DELAY solves the problems. For more hints read

the (pseudo) FAQ and the documentation coming with z8530drv-utils.

I got reports that the driver has problems on some 386-based systems.

(i.e. Amstrad) Those systems have a bogus AT bus timing which will

lead to delayed answers on interrupts. You can recognize these

problems by looking at the output of Sccstat for the suspected

port. See if it shows under- and overruns you own such a system.

Delayed processing of received data: This depends on

- the kernel version

- kernel profiling compiled or not

- a high interrupt load

- a high load of the machine --- running X, Xmorph, XV and Povray,

while compiling the kernel... hmm ... even with 32 MB RAM ...  ;-)

Or running a named for the whole .ampr.org. domain on an 8 MB

box...

- using information from rxecho or kissbridge.

Kernel panics: please read to /linux/README and find out if it

really occurred within the scc driver.

If you cannot solve a problem, send me

- a description of the problem,

- information on your hardware (computer system, scc board, modem)

- your kernel version

- the output of cat /proc/net/z8530

4. Thor RLC100

==============

Mysteriously this board seems not to work with the driver. Anyone

got it up-and-running?

5. Appendix

===========

The fullduplex mode 3 works as follows:

1. The protocol layer sends a KISS command

[PARAM_RTS] [TX_ON]

to key up the transmitter. Then it sends the data frame(s) to

the driver. When all frames are sent, the driver will send the

KISS command

[PARAM_HWEVENT] [HWEV_ALL_SENT].

The protocol layer may now key down the transmitter with

[PARAM_RTS] [TX_OFF]

or send more frames. Note that the maxkeyup timer may expire and

key down the transceiver before everything is sent.

2. The driver sends

[PARAM_HWEVENT] [HWEV_DCD_ON]

or

[PARAM_HWEVENT] [HWEV_DCD_OFF]

if the status of the DCD changes.

3. The protocol layer can send

[PARAM_HWEVENT] 0

the driver will reply with one of the DCD status messages.

Note that KISS command doesn't necessarily mean that the enclosed data

is SLIP encoded. In network driver mode the driver does not encode/decode

SLIP, but it will still distinguish between a data and a command packet

by the leading byte, just like in 'real' KISS mode.

Note that this feature may vanish without a notice.

6. Thanks

=========

Many thanks to everyone who contributed to this project with ideas,

error reports and bug fixes. Special thanks to Linus Torvalds and Alan

Cox for including the driver in the Linux standard distribution and

their support.

Joerg Reuter	ampr-net: dl1bke@db0pra.ampr.org

AX-25   : DL1BKE @ DB0ACH.#NRW.DEU.EU

Internet: jreuter@lykos.oche.de

WWW     : http://poboxes.com/jreuter/