The  compressor achieves  an  average compression  rate of 60%  of the

original size which is on par with "gzip". It seems that you cannot do

much better for compressing  compiled  binaries.  This means that  the

break even  point  for using compressed  images is   reached, once the

uncompressed size approaches 1.5kB. We  can stuff more than 12kB  into

an 8kB EPROM and more than 25kB into an 16kB EPROM.   As there is only

32kB of RAM  for both the uncompressed  image  and its BSS  area, this

means that 32kB EPROMs will hardly ever be required.

The compression  algorithm uses a  4kB  ring buffer  for buffering the

uncompressed data. Before   compression starts,  the  ring buffer   is

filled  with spaces (ASCII  character  0x20).  The algorithm tries  to

find repeated  input sequences of a  maximum length of  60 bytes.  All

256 different input  bytes  plus the 58 (60   minus a threshold of  2)

possible  repeat lengths form a set  of 314 symbols. These symbols are

adaptively Huffman encoded.  The  algorithm starts out with a Huffmann

tree  that  assigns equal code lengths    to each of  the  314 symbols

(slightly favoring the repeat  symbols over symbols for regular  input

characters), but  it will be changed whenever  the frequency of any of

the symbols  changes. Frequency counts are  kept in 16bit  words until

the total number of compressed codes totals 2^15.  Then, all frequency

counts will be halfed (rounding to the bigger number).  For unrepeated

characters (symbols 0..255) the Huffman code  is written to the output

stream.  For repeated characters the  Huffmann code, which denotes the

length of the repeated character sequence, is written out and then the

index in the ring buffer is computed.   From this index, the algorithm

computes  the offset   relative to  the current  index  into  the ring

buffer. Thus,  for typical input data,  one would expect that short to

medium range offsets are more frequent  than extremely short or medium

range to long range offsets. Thus the  12bit (for a 4kB buffer) offset

value  is statically Huffman encoded  using a precomputed Huffman tree

that favors  those  offset  values    that  are deemed to   be    more

frequent. The  Huffman encoded offset  is  written to the output  data

stream,  directly  following the code  that   determines the length of

repeated characters.

This algorithm, as implemented in the  C example code, looks very good

and  its operating parameters are   already well optimized. This  also

explains   why  it achieves     compression ratios    comparable  with

"gzip". Depending on the input data, it sometimes excells considerably

beyond what "gzip -9" does, but this  phenomenon does not appear to be

typical. There are some flaws with  the algorithm, such as the limited

buffer  sizes, the  adaptive  Huffman tree  which takes  very  long to

change, if    the input  characters  experience   a sudden   change in

distribution, and the static Huffman   tree for encoding offsets  into

the  buffer.   The slow  changes of   the  adaptive  Huffman  tree are

partially counteracted by  artifically keeping  a 16bit precision  for

the frequency counts, but  this does not  come into play until 32kB of

compressed data is output, so  it does not  have any impact on our use

for "etherboot", because  the BOOT Prom  does not support uncompressed

data of more then 32kB (c.f. doc/spec.doc).

Nonetheless,  these problems  do  not  seem  to affect  compression of

compiled  programs very much.  Mixing  object code with English  text,

would not work too  well though, and  the algorithm should be reset in

between. Actually, we  might  gain a little  improvement, if  text and

data   segments    were compressed  individually,    but   I have  not

experimented with this option, yet.